FPGA-Based Edge-AI Vision Accelerator for Streaming CNN Convolution

IEEE Solid-State Circuits Society (SSCS) Egypt Chapter 2026 Student Design Competition

Team members:

Farah Ihab

Manar Saber

Mai Hamed

Naira Wasseem

Table of Contents

1.0 Abstract	1

2.0 Introduction	2

3.0 Design Specifications & Compliance Summary	2

4.0 Overall Architecture and Dataflow	3

5.0 Input Streaming and Memory Organization	3

6.0 Sliding Window Generation	4

7.0 Precision and Bit-Width Growth Analysis	4

8.0 MAC Datapath and Pipelining	4

9.0 Activation and Output Saturation	5

10.0 Output Buffering and Continuous Throughput	5

11.0 Verification Methodology and Results	5

12.0 FPGA Implementation and Physical Results	5

13.0 Timing Closure Analysis	6

14.0 Power Analysis	6

15.0 Figure of Merit (FoM) Calculation	7

16.0 Design Trade-offs and Architectural Discussion	7

17.0 Conclusion and Future Work	7

# 1.0 Abstract

This report presents the design, implementation, and verification of a resource-efficient FPGA-based Edge-AI vision accelerator for two-dimensional CNN convolution. The proposed accelerator processes a minimum 32 × 32 single-channel grayscale image using an 8-bit unsigned input representation and a programmable 3 × 3 kernel with 8-bit signed coefficients. The architecture employs a streaming datapath incorporating two line buffers and a sliding-window generator to construct 3 × 3 convolution windows on the fly without storing the complete image frame on-chip. Nine parallel multiply-accumulate (MAC) operations execute the spatial convolution, followed by an optional ReLU activation unit and a saturation stage that maps the internal 20-bit signed accumulator result to a 16-bit signed output. Pipelining ensures timing closure at high operating frequencies, while an output FIFO absorbs row-boundary spatial bubbles to deliver a sustained throughput of one output pixel per clock cycle. RTL verification against an independent Python golden reference confirmed functional equivalence across 8,100 output pixels with zero mismatches. Synthesized and implemented on a Xilinx Zynq-7000 XC7Z020 FPGA, the design utilizes 1,501 LUTs, 2,552 flip-flops, 0 DSP blocks, and 0 BRAM tiles. The accelerator closes timing up to 162.5 MHz (with 160 MHz recommended). Total estimated on-chip power is 0.168 W, achieving a competition Figure of Merit (FoM) of 3.97 × 10⁻³.

# 2.0 Introduction

Convolutional Neural Networks (CNNs) serve as the foundation for modern edge vision applications, including real-time image classification, robotic perception, and industrial defect detection. However, 2D spatial convolutions represent the primary computational bottleneck in deep vision pipelines due to their high multiply-accumulate (MAC) density. Deploying deep networks to resource- and thermal-constrained edge devices necessitates dedicated hardware architectures optimized for area, power, and sustained throughput.

Field-Programmable Gate Arrays (FPGAs) offer an ideal substrate for edge acceleration, enabling fine-grained parallelization, deep spatial pipelining, and custom bit-width quantization without the recurring non-engineering costs of dedicated ASICs. Rather than retrieving pixel matrices sequentially via memory-bus interfaces, streaming architectures can process raster data directly as it arrives.

The objective of this project is to implement a compact FPGA-based CNN convolution accelerator satisfying the requirements of the IEEE Solid State Circuits Society (SSCS) Egypt Chapter 2026 Student Design Competition. The design targets a minimum 32 × 32 grayscale image or feature map, an 8-bit unsigned input representation, an 8-bit signed programmable convolution kernel, stride one operation, and a minimum 16-bit signed output.

Design Objectives

Efficient Raster Streaming: Cascaded line buffers eliminate full-frame on-chip storage.

High Computational Parallelism: Nine parallel MAC operations execute spatial 3 × 3 convolution simultaneously.

Pipelined Datapath: Balanced 5-stage adder tree enables timing closure at 160 MHz.

Zero DSP / BRAM Penalty: 100% logic-based arithmetic avoids heavy penalty factors in competition FoM.

Continuous 1.0 Pixel/Cycle Throughput: Output FIFO absorbs horizontal blanking gaps.

Arithmetic Integrity: 20-bit internal accumulator with symmetric 16-bit saturation avoids wraparound.

Golden Model Verification: 100% equivalence verified across 8,100 pixels against an independent Python model.

# 3.0 Design Specifications & Compliance Summary

Table 1 outlines the contest specifications alongside the implementation metrics achieved by the team.

| Parameter | Contest Spec | Team Result | Units | Implementation Details / Architectural Notes |
| --- | --- | --- | --- | --- |
| Input Image Size | Min. 32 × 32 | 32 × 32 | pixels | Single-channel grayscale raster stream |
| Input Precision | Unsigned fixed-point | 8 | bits | Unsigned integer format ([0, 255]) |
| Kernel Size | N × N | 3 × 3 | — | 9 configurable coefficients |
| Kernel Precision | 8-bit signed | 8 | bits | Two's complement integer ([-128, 127]) |
| Stride | 1 | 1 | — | Unit stride, no row/column skipping |
| Output Precision | Min. 16-bit signed | 16 | bits | Signed output with symmetric saturation |
| Activation Function | Optional ReLU | Supported | — | Configurable post-accumulation bypass |
| Architecture Type | FPGA CNN Accelerator | Streaming Pipelined | — | Line buffers, sliding window, parallel MAC |
| Multipliers / MACs | Contestant choice | 9 | MACs | Fully parallelized spatial arithmetic |
| Pipeline Stages | Contestant choice | 5 | cycles | Balanced pipelined adder-tree datapath |
| Throughput | Output pixels / cycle | 1.0 | pixel/cyc | Continuous valid output stream (bonus) |
| Recommended Freq. | High frequency | 160.0 | MHz | Robust timing margin preserved (WNS = +0.131 ns) |
| Maximum Verified Freq. | Fmax closure | 162.5 | MHz | Zero failing endpoints (WNS = +0.010 ns) |
| LUT Utilization | Minimize | 1,501 | LUTs | 2.82% of XC7Z020 capacity |
| FF Utilization | Minimize | 2,552 | FFs | 2.40% of XC7Z020 capacity |
| DSP Utilization | Report count | 0 | DSPs | 100% LUT-based arithmetic (0 penalty in FoM) |
| BRAM Utilization | Report count | 0 | BRAMs | Distributed shift-register line storage (0 penalty) |
| Power Estimate | Minimize | 0.168 | W | Vivado vectorless power analysis |
| Verification Status | Golden model check | 8,100 / 8,100 | outputs | 100% match, zero reported mismatches |
| Figure of Merit (FoM) | Higher is better | 3.97 × 10⁻³ | — | Evaluated per contest formula: FoM = Tp / (P × Logic) |

# 4.0 Overall Architecture and Dataflow

The accelerator uses a streaming architecture in which input pixels are supplied sequentially rather than loading the entire image into an on-chip frame buffer. The major functional blocks operate synchronously:

Dataflow Pipeline Sequence:
Input Stream [8-bit] ──> Line Buffers ──> 3×3 Window Gen ──> 9× Parallel MAC ──> ReLU ──> Saturation ──> Result FIFO ──> Pixel Out [16-bit]
                                                               ▲
                                   Kernel Registers (9 × 8-bit) ─┘

The kernel coefficients are supplied through the configurable kernel interface and stored internally in kernel registers. During processing, each incoming pixel contributes to a sliding 3 × 3 window. The window is flattened into nine values and supplied to the MAC datapath together with the nine kernel coefficients. The arithmetic result passes through the optional ReLU stage and is subsequently saturated from the internal 20-bit accumulator width to the required 16-bit signed output width.

# 5.0 Input Streaming and Memory Organization

A major architectural objective was to avoid storing the complete 32 × 32 image on-chip. For a 3 × 3 convolution, each output requires pixels from three consecutive image rows. Therefore, only the previous two rows need to be retained in addition to the current input stream.

The design uses two line buffers arranged in cascade. The first line buffer stores and provides pixels from the previous row (r - 1), while the second provides pixels from two rows earlier (r - 2). The current input pixel (r) is provided directly from the input stream. Conceptually, the line-buffer subsystem provides three simultaneous vertical pixels to the window generator.

Synthesizing these line buffers using distributed shift registers (SRL16E / SRL32E) rather than dedicated block RAM primitives keeps dedicated BRAM utilization at exactly zero, preventing the 100× penalty associated with BRAMs in the competition Figure of Merit.

# 6.0 Sliding Window Generation

The convolution engine requires a 3 × 3 spatial window for every valid output pixel. The window generator combines the three pixel streams from the line-buffer subsystem with internal shift registers:

W = [ x₀  x₁  x₂ ;  x₃  x₄  x₅ ;  x₆  x₇  x₈ ] = [ X[r-2, c-2]  X[r-2, c-1]  X[r-2, c] ;  X[r-1, c-2]  X[r-1, c-1]  X[r-1, c] ;  X[r, c-2]  X[r, c-1]  X[r, c] ]

As each new pixel arrives, the registers shift previously received pixels and insert the new pixel into the active column. Because the first two image rows and the first two columns of each row do not contain three spatial neighbors, window_valid is asserted only when two complete previous rows are available and the current column is sufficiently advanced (r ≥ 2, c ≥ 2). For a 32 × 32 input matrix, the valid spatial output dimension is (32 - 3 + 1) × (32 - 3 + 1) = 30 × 30 = 900 valid output pixels.

# 7.0 Precision and Bit-Width Growth Analysis

Given unsigned 8-bit input pixels xᵢ ∈ [0, 255] and signed 8-bit kernel weights wᵢ ∈ [-128, 127], each individual product pᵢ = xᵢ × wᵢ spans the numerical range:

• p_min = 255 × (-128) = -32,640
• p_max = 255 × 127 = 32,385

A 16-bit signed integer (range [-32,768, 32,767]) is sufficient to accommodate each multiplication without precision loss. Summing all nine spatial products produces:

• y_min = 9 × 255 × (-128) = -293,760
• y_max = 9 × 255 × 127 = +291,465

Representing these bounds requires ⌈log₂(293,760 + 1)⌉ + 1 = 20 bits in two's-complement notation (covering [-524,288, +524,287]). Thus, an internal 20-bit accumulator guarantees zero internal overflow.

# 8.0 MAC Datapath and Pipelining

The core convolution uses nine parallel multiplier operations implemented in logic LUTs, followed by a balanced adder tree. To achieve timing closure above 160 MHz, the arithmetic datapath is partitioned across five pipeline stages:

Stage 1: Registering spatial window inputs and kernel coefficients.

Stage 2: 9 parallel signed-by-unsigned 8 × 8 multiplier blocks implemented in LUTs.

Stage 3: Primary adder tree reducing nine 16-bit products into intermediate partial sums.

Stage 4: Secondary accumulation stage generating the full 20-bit sum.

Stage 5: Configurable ReLU masking and symmetric 16-bit saturation logic.

Control signals (valid_in) propagate through an identical 5-stage shift register to ensure that the output-valid signal remains strictly aligned with the computed data.

# 9.0 Activation and Output Saturation

The accelerator includes an optional Rectified Linear Unit (ReLU) stage: ReLU(y) = max(0, y). When enabled, negative convolution results are clamped to zero. When disabled, the signed convolution sum is preserved.

The competition requires a minimum 16-bit signed output Y_out ∈ [-32,768, +32,767]. Rather than truncating upper bits (which causes catastrophic wraparound), symmetric saturation is applied: if y > 32,767, Y_out = 32,767; if y < -32,768, Y_out = -32,768; otherwise Y_out = y[15:0].

# 10.0 Output Buffering and Continuous Throughput

The sliding-window architecture naturally produces 2-cycle bubbles at horizontal image boundaries because the first two columns of each row cannot generate valid 3 × 3 windows. To achieve the competition's bonus feature for a continuous 1-pixel/cycle pipeline, a result FIFO is placed after the saturation stage. The FIFO buffers valid convolution results and streams them out continuously once a sufficient threshold has accumulated. Tested across the full 30 × 30 output map, the accelerator delivered all 900 valid output pixels over 900 consecutive clock cycles with zero gaps, achieving a sustained throughput of exactly 1.0 pixel/cycle.

# 11.0 Verification Methodology and Results

The RTL was verified against an independent Python golden reference model executing bit-accurate arithmetic. Test vectors included normal convolution with ReLU, extreme saturation tests (y = -293,760 → -32,768 and y = +291,465 → +32,767), identity matrices, and non-continuous input stalls. Across 9 test images and 8,100 evaluated output pixels, all 8,100 pixels matched the reference model with zero errors.

# 12.0 FPGA Implementation and Physical Results

The design was synthesized and implemented in Vivado 2018.2 targeting a Xilinx Zynq-7000 SoC (xc7z020clg400-1). Table 2 details the device utilization, and Table 3 presents the hierarchical submodule breakdown.

| Resource Type | Used | Available | Utilization (%) | Notes / Architectural Impact |
| --- | --- | --- | --- | --- |
| Slice LUTs | 1,501 | 53,200 | 2.82% | Low logic footprint; accommodates entire MAC tree |
| — LUT as Logic | 1,468 | 53,200 | 2.76% | Combinational logic and adder tree |
| — LUT as Memory | 33 | 17,400 | 0.19% | Distributed shift-register line storage |
| Slice Registers (FFs) | 2,552 | 106,400 | 2.40% | Pipeline registers and result FIFO storage |
| Occupied Slices | 1,164 | 13,300 | 8.75% | Compact spatial footprint |
| F7 Muxes | 272 | 26,600 | 1.02% | Result FIFO address multiplexing |
| F8 Muxes | 128 | 13,300 | 0.96% | Result FIFO address multiplexing |
| Bonded IOBs | 102 | 125 | 81.60% | Parallel testbench I/O configuration |
| Clock Buffers (BUFG) | 1 | 32 | 3.13% | Single global clock tree |
| DSP48E1 Blocks | 0 | 220 | 0.00% | Zero penalty in competition FoM denominator |
| Block RAM (BRAM) | 0 | 140 | 0.00% | Zero penalty in competition FoM denominator |

| Submodule Instance | LUTs | FFs | Slices | F7 Mux | F8 Mux | Description / Sub-function |
| --- | --- | --- | --- | --- | --- | --- |
| u_controller | 2 | 6 | 4 | 0 | 0 | Global control FSM and valid management |
| u_pipelined_mac | 543 | 303 | 238 | 0 | 0 | 9-way parallel multiplier and adder tree |
| u_result_fifo | 717 | 2,081 | 884 | 272 | 128 | Decoupling FIFO for 1.0 pixel/cycle throughput |
| u_window_gen | 275 | 90 | 125 | 0 | 0 | Cascaded line buffers and sliding window |
| Total Top Level | 1,501 | 2,552 | 1,164 | 272 | 128 | Fully closed, zero DRC/timing violations |

# 13.0 Timing Closure Analysis

Implementation runs were evaluated across clock constraints ranging from 100 MHz down to 6.154 ns (162.5 MHz). Table 4 details setup, hold, and pulse width slacks.

| Target Frequency | Clock Period | WNS | WHS | WPWS | Failing Endpoints | Timing Status |
| --- | --- | --- | --- | --- | --- | --- |
| 100.0 MHz | 10.000 ns | +2.525 ns | +0.070 ns | 3.750 ns | 0 | PASS (Baseline run) |
| 125.0 MHz | 8.000 ns | +1.504 ns | +0.035 ns | 2.750 ns | 0 | PASS (Intermediate sweep) |
| 150.0 MHz | 6.667 ns | +0.446 ns | +0.144 ns | 2.083 ns | 0 | PASS (Intermediate sweep) |
| 160.0 MHz | 6.250 ns | +0.131 ns | +0.119 ns | 1.875 ns | 0 | PASS (Recommended operating point) |
| 162.5 MHz | 6.154 ns | +0.010 ns | +0.085 ns | 1.827 ns | 0 | PASS (Maximum verified frequency) |

The design closes timing up to 162.5 MHz with zero failing endpoints (WNS = +0.010 ns). 160.0 MHz was selected as the recommended operating frequency (WNS = +0.131 ns) to provide safe guardbands against PVT variations. At 160 MHz and 1.0 pixel/cycle, sustained throughput reaches 160 million output pixels per second.

# 14.0 Power Analysis

Power estimation was performed using Vivado Power Analysis on the fully routed netlist (Table 5).

| Parameter / Power Domain | Value | Percentage | Thermal and Environmental Conditions |
| --- | --- | --- | --- |
| Total On-Chip Power | 0.168 W | 100.0% | Ambient: 25.0°C |
| — Dynamic Power | 0.063 W | 37.5% | Active switching power |
| — Device Static Power | 0.105 W | 62.5% | Quiescent leakage power |
| Signal Routing Power | 0.027 W | 42.9% | Routing network interconnect capacitance |
| Logic Core Power | 0.014 W | 22.2% | Slice LUT switching activity |
| I/O Buffer Power | 0.014 W | 22.2% | Pin driver transitions |
| Clock Tree Power | 0.008 W | 12.7% | Global clock distribution networks |
| Junction Temperature | 26.9°C | — | Safe thermal operating boundary |
| Thermal Margin | 58.1°C | — | Safe margin (4.9 W dissipation limit) |
| Effective Thermal Resistance (θ_JA) | 11.5°C/W | — | Standard package thermal profile |

# 15.0 Figure of Merit (FoM) Calculation

The competition evaluates accelerator efficiency using the normalized Figure of Merit:

FoM = Throughput / [ Power × (LUTs + 50 × DSPs + 100 × BRAMs) ]
FoM = 1.0 / [ 0.168 × (1,501 + 50 × 0 + 100 × 0) ]
FoM = 1.0 / [ 0.168 × 1,501 ] = 1.0 / 252.168 ≈ 3.9656 × 10⁻³  (3.97 × 10⁻³)

By eliminating dedicated DSPs and Block RAMs, the design completely avoids denominator penalty factors, achieving a competitive FoM score of 3.97 × 10⁻³.

# 16.0 Design Trade-offs and Architectural Discussion

LUT Arithmetic vs. Dedicated DSPs: Implementing nine 8 × 8 multipliers directly in slice LUTs required 543 LUTs. In exchange, eliminating DSP blocks avoided a penalty of 9 × 50 = 450 effective units in the contest FoM denominator.

Cascaded Line Buffers vs. BRAM Frame Storage: Raster line buffering eliminates whole-frame storage requirements, reducing RAM overhead to zero BRAM primitives and avoiding a 100× penalty.

Output FIFO Overhead vs. Sustained Throughput: The output buffer consumes 717 LUTs and 2,081 FFs, but increases sustained throughput from ≈ 0.88 to 1.0 pixel/cycle, directly improving the FoM numerator.

Precision Optimization: An internal 20-bit accumulator provides complete dynamic range without overflow, while 16-bit output saturation satisfies contest interface constraints.

# 17.0 Conclusion and Future Work

An optimized, streaming FPGA vision accelerator for 2D CNN convolution was successfully designed, verified, and implemented on a Xilinx Zynq-7000 SoC. By combining cascaded line buffering, zero-DSP pipelined arithmetic, symmetric saturation, and continuous output FIFO scheduling, the design achieves 160 MHz timing closure, 100% golden-model verification across 8,100 output pixels, 1.0 pixel/cycle throughput, and a competitive Figure of Merit of 3.97 × 10⁻³.

Future enhancements include extending the architecture to support reconfigurable kernel sizes (5 × 5, 7 × 7), integrating multi-channel feature maps, and conducting physical board-level power characterization.